P 1 Single-Area OSPF Link Costs and Interface Priorities.

Objectives

• Configure single-area OSPF on a router.

• Advertise loopback interfaces into OSPF.

• Verify OSPF adjacencies.

• Verify OSPF routing information exchange.

• Modify OSPF link costs.

• Change interface priorities.

Step 1: Configure addressing and loopbacks.

R1

R1# configure terminal

R1(config)# interface Loopback1

R1(config-if)# description Engineering Department

R1(config-if)# ip address 10.1.1.1 255.255.255.0

R1(config-if)# exit

R1(config)# interface FastEthernet0/0

R1(config-if)# ip address 10.1.200.1 255.255.255.0

R1(config-if)# no shutdown

R1(config-if)# exit

R1(config)# interface Serial 0/0/0

R1(config-if)# ip address 10.1.100.1 255.255.255.0

R1(config-if)# clock rate 64000

R1(configif)# bandwidth 64

R1(config-if)# no shutdown

R2

R2# configure terminal

R2(config)# interface Loopback2

R2(config-if)# description Marketing Department

R2(config-if)# ip address 10.1.2.1 255.255.255.0

R2(config-if)# exit

R2(config)# interface FastEthernet0/0

R2(config-if)# ip address 10.1.200.2 255.255.255.0

R2(config-if)# no shutdown

R2(config-if)# exit

R2(config)# interface Serial 0/0/0

R2(config-if)# ip address 10.1.100.2 255.255.255.0

R2(config-if)# bandwidth 64

R2(config-if)# no shutdown

R3

R3# configure terminal

R3(config)# interface Loopback3

R3(config-if)# description Accounting Department

R3(config-if)# ip address 10.1.3.1 255.255.255.0

R3(config-if)# exit

R3(config)# interface FastEthernet0/0

R3(config-if)# ip address 10.1.200.3 255.255.255.0

R3(config-if)# no shutdown

R3(config-if)# exit

Step 2: Add physical interfaces to OSPF.

R1

R1(config)# router ospf 1

R1(config-router)# network 10.1.100.0 0.0.0.255 area 0

R1(config-router)# network 10.1.200.0 0.0.0.255 area 0

R1(config-router)# end

R2

R2(config)# router ospf 1

R2(config-router)# network 10.1.100.0 0.0.0.255 area 0

R2(config-router)# network 10.1.200.0 0.0.0.255 area 0

R3

R3(config)# router ospf 1

R3(config-router)# network 10.1.200.0 0.0.0.255 area 0

Step 3: Use OSPF show commands.

R1

R1# show ip protocols

R1# show ip ospf

R1# show ip ospf neighbor

R1# show ip ospf interface FastEthernet 0/0

R1# show ip ospf interface brief

R1# show ip ospf database

Step 4: Add loopback interfaces to OSPF.

R1# show ip route

R2# show ip route

R3# show ip route

R1

R1(config)# router ospf 1

R1(config-router)# network 10.1.1.0 0.0.0.255 area 0

R2

R2(config)# router ospf 1

R2(config-router)# network 10.1.2.0 0.0.0.255 area 0

R3

R3(config)# router ospf 1

R3(config-router)# network 10.1.3.0 0.0.0.255 area 0

R1# show ip route

R2# show ip route

R3# show ip route

R1

R1(config)# interface loopback1

R1(config-if)# ip ospf network point-to-point

R2

R2(config)# interface loopback2

R2(config-if)# ip ospf network point-to-point

R3

R3(config)# interface loopback3

R3(config-if)# ip ospf network point-to-point

Step 5: Modify OSPF link costs.

R1

R1(config)# interface FastEthernet 0/0

R1(config-if)# ip ospf cost 50

R2

R2(config)# interface FastEthernet 0/0

R2(config-if)# ip ospf cost 50

R3

R3(config)# interface FastEthernet 0/0

R3(config-if)# ip ospf cost 50

R1# show ip route

Step 6: Modify interface priorities to control the DR and BDR election.

R1

R1(config)# interface FastEthernet 0/0

R1(config-if)# ip ospf priority 10

R2

R2(config)# interface FastEthernet 0/0

R2(config-if)# ip ospf priority 5

R1

R1# show ip ospf neighbor detail

P 2 Multi-Area OSPF with Stub Areas and Authentication

Step 1: Configure addressing and loopbacks.

R1# configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

R1(config)# interface loopback 1

R1(config-if)# description Engineering Department

R1(config-if)# ip address 10.1.1.1 255.255.255.0

R1(config-if)# interface serial 0/0/0

R1(config-if)# ip address 10.1.12.1 255.255.255.0

R1(config-if)# clockrate 64000

R1(config-if)# no shutdown

R2# configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

R2(config)# interface loopback 2

R2(config-if)# description Marketing Department

R2(config-if)# ip address 10.1.2.1 255.255.255.0

R2(config-if)# interface serial 0/0/0

R2(config-if)# ip address 10.1.12.2 255.255.255.0

R2(config-if)# no shutdown

R2(config-if)# interface serial 0/0/1

R2(config-if)# ip address 10.1.23.2 255.255.255.0

R2(config-if)# clockrate 64000

R2(config-if)# no shutdown

R3# configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

R3(config)# interface loopback 3

R3(config-if)# description Accounting Department

R3(config-if)# ip address 10.1.3.1 255.255.255.0

R3(config-if)# interface loopback 20

R3(config-if)# description Connection to another AS

R3(config-if)# ip address 172.20.200.1 255.255.255.0

R3(config-if)# interface serial 0/0/1

R3(config-if)# ip address 10.1.23.3 255.255.255.0

R3(config-if)# no shutdown

Step 2: Add interfaces into OSPF.

R1(config)# router ospf 1

R1(config-router)# network 10.1.12.0 0.0.0.255 area 0

R1(config-router)# network 10.1.1.0 0.0.0.255 area 0

R1(config-router)# exit

R1(config)# interface loopback 1

R1(config-if)# ip ospf network point-to-point

R2(config)# router ospf 1

R2(config-router)# network 10.1.12.0 0.0.0.255 area 0

R2(config-router)# network 10.1.2.0 0.0.0.255 area 0

R2(config-router)# exit

R2(config)# interface loopback 2

R2(config-if)# ip ospf network point-to-point

Add the subnet between R2 and R3 into OSPF area 23 using the network command. Add loopback 3 on

R3 into area 23.

R2(config)# router ospf 1

R2(config-router)# network 10.1.23.0 0.0.0.255 area 23

R3(config)# router ospf 1

R3(config-router)# network 10.1.23.0 0.0.0.255 area 23

R3(config-router)# network 10.1.3.0 0.0.0.255 area 23

R3(config-router)# exit

R3(config)# interface loopback 3

R3(config-if)# ip ospf network point-to-point

Step 3: Configure a stub area.

R2(config)# router ospf 1

R2(config-router)# area 23 stub

R3(config)# router ospf 1

R3(config-router)# area 23 stub

Step 4: Configure a totally stubby area.

R2(config)# router ospf 1

R2(config-router)# area 23 stub no-summary

Step 5: Configure OSPF interface authentication.

R2(config)# interface serial 0/0/1

R2(config-if)# ip ospf authentication

R2(config-if)# ip ospf authentication-key cisco

R3(config)# interface serial 0/0/1

R3(config-if)# ip ospf authentication

R3(config-if)# ip ospf authentication-key cisco

R1(config)# interface serial 0/0/0

R1(config-if)# ip ospf authentication message-digest

R1(config-if)# ip ospf message-digest-key 1 md5 cisco

R2(config)# interface serial 0/0/0

R2(config-if)# ip ospf authentication message-digest

R2(config-if)# ip ospf message-digest-key 1 md5 cisco

P 3 Redistribution Between RIP and OSPF

Step 1: Configure loopbacks and assign addresses.

R1(config)# interface Loopback0

R1(config-if)# ip address 172.16.1.1 255.255.255.0

R1(config-if)# interface Loopback48

R1(config-if)# ip address 192.168.48.1 255.255.255.0

R1(config-if)# interface Loopback49

R1(config-if)# ip address 192.168.49.1 255.255.255.0

R1(config-if)# interface Loopback50

R1(config-if)# ip address 192.168.50.1 255.255.255.0

R1(config-if)# interface Loopback51

R1(config-if)# ip address 192.168.51.1 255.255.255.0

R1(config-if)# interface Loopback70

R1(config-if)# ip address 192.168.70.1 255.255.255.0

R1(config-if)# interface Serial0/0/0

R1(config-if)# ip address 172.16.12.1 255.255.255.0

R1(config-if)# clock rate 64000

R1(config-if)# bandwidth 64

R1(config-if)# no shutdown

R2(config)# interface Loopback0

R2(config-if)# ip address 172.16.2.1 255.255.255.0

R2(config-if)# interface Serial0/0/0

R2(config-if)# ip address 172.16.12.2 255.255.255.0

R2(config-if)# bandwidth 64

R2(config-if)# no shutdown

R2(config-if)# interface Serial0/0/1

R2(config-if)# ip address 172.16.23.2 255.255.255.0

R2(config-if)# clock rate 64000

R2(config-if)# bandwidth 64

R2(config-if)# no shutdown

R3(config)# interface Loopback0

R3(config-if)# ip address 172.16.3.1 255.255.255.0

R3(config-if)# interface Loopback20

R3(config-if)# ip address 192.168.20.1 255.255.255.0

R3(config-if)# interface Loopback25

R3(config-if)# ip address 192.168.25.1 255.255.255.0

R3(config-if)# interface Loopback30

R3(config-if)# ip address 192.168.30.1 255.255.255.0

R3(config-if)# interface Loopback35

R3(config-if)# ip address 192.168.35.1 255.255.255.0

R3(config-if)# interface Loopback40

R3(config-if)# ip address 192.168.40.1 255.255.255.0

R3(config-if)# interface Serial0/0/1

R3(config-if)# ip address 172.16.23.3 255.255.255.0

R3(config-if)# bandwidth 64

R3(config-if)# no shutdown

Step 2: Configure RIPv2.

R1(config)# router rip

R1(config-router)# version 2

R1(config-router)# no auto-summary

R1(config-router)# network 172.16.0.0

R1(config-router)# network 192.168.48.0

R1(config-router)# network 192.168.49.0

R1(config-router)# network 192.168.50.0

R1(config-router)# network 192.168.51.0

R1(config-router)# network 192.168.70.0

R2(config)# router rip

R2(config-router)# version 2

R2(config-router)# no auto-summary

R2(config-router)# network 172.16.0.0

R1# show ip route rip

R2# show ip route rip

Step 3: Configure passive interfaces in RIP.

R1# show ip route rip

R2(config)# router rip

R2(config-router)# passive-interface serial 0/0/1

R2# show ip protocols

R1(config)# router rip

R1(config-router)# passive-interface loopback 0

R1(config-router)# passive-interface loopback 48

R1(config-router)# passive-interface loopback 49

R1(config-router)# passive-interface loopback 50

R1(config-router)# passive-interface loopback 51

R1(config-router)# passive-interface loopback 70

R2(config)# router rip

R2(config-router)# passive-interface loopback 0

Step 4: Summarize a supernet with RIP.

R2# show ip route rip

R1(config)# ip route 192.168.48.0 255.255.252.0 null0

R1(config)# router rip

R1(config-router)# redistribute static

R1# show ip route

R2# show ip route

Step 5: Suppress routes using prefix lists.

R1(config)# ip prefix-list RIP-OUT permit 192.168.48.0/22

R1(config)# ip prefix-list RIP-OUT deny 192.168.48.0/22 le 24

R1(config)# ip prefix-list RIP-OUT permit 0.0.0.0/0 le 32

R1(config)# router rip

R1(config-router)# distribute-list prefix RIP-OUT out serial0/0/0

R2# show ip route rip

R2# show ip rip database

Step 6: Configure OSPF.

R2(config)# router ospf 1

R2(config-router)# network 172.16.23.0 0.0.0.255 area 0

R3(config)# router ospf 1

R3(config-router)# network 172.16.0.0 0.0.255.255 area 0

R3(config-router)# network 192.168.0.0 0.0.255.255 area 0

R3(config)# interface Loopback0

R3(config-if)# ip ospf network point-to-point

R3(config-if)# interface Loopback20

R3(config-if)# ip ospf network point-to-point

R3(config-if)# nterface Loopback25

R3(config-if)# ip ospf network point-to-point

R3(config-if)# interface Loopback30

R3(config-if)# ip ospf network point-to-point

R3(config-if)# interface Loopback35

R3(config-if)# ip ospf network point-to-point

R3(config-if)# interface Loopback40

R3(config-if)# ip ospf network point-to-point

R3# show ip ospf neighbor

R2# show ip route ospf

R3# show ip route ospf

Step 7: Configure passive interfaces in OSPF.

R3(config)# router ospf 1

R3(config-router)# passive-interface default

R3(config-router)# no passive-interface serial 0/0/1

R3# how ip protocols

Step 8: Allow one-way redistribution.

R2(config)# router rip

R2(config-router)# redistribute ospf 1 metric 4

R2# show ip protocols

R1# show ip route rip

R1# ping 192.168.30.1

R1# traceroute 192.168.30.1

R2(config)# router ospf 1

R2(config-router)# default-information originate always

Step 9: Redistribute between two routing protocols.

R2(config)# router ospf 1

R2(config-router)# no default-information originate always

R2(config-router)# redistribute rip

R3# show ip route ospf

R2(config)# router ospf 1

R2(config-router)# redistribute rip subnets

R3# show ip route ospf

P 4 Manipulating Administrative Distances

Step 1: Configure router loopbacks and addressing.

R1# conf t

R1(config)# interface loopback 1

R1(config-if)# ip address 172.16.1.1 255.255.255.0

R1(config-if)# interface loopback 101

R1(config-if)# ip address 192.168.101.1 255.255.255.0

R1(config-if)# interface fastethernet 0/0

R1(config-if)# ip address 172.16.12.1 255.255.255.0

R1(config-if)# no shutdown

R1(config-if)# interface serial 0/0/1

R1(config-if)# bandwidth 64

R1(config-if)# ip address 172.16.13.1 255.255.255.0

R1(config-if)# no shutdown

R2# conf t

R2(config)# interface loopback 2

R2(config-if)# ip address 172.16.2.1 255.255.255.0 R2(configif)#

interface loopback 102

R2(config-if)# ip address 192.168.102.1 255.255.255.0

R2(config-if)# interface fastethernet 0/0

R2(config-if)# ip address 172.16.12.2 255.255.255.0

R2(config-if)# no shutdown

R2(config-if)# interface fastethernet 0/1

R2(config-if)# ip address 172.16.23.2 255.255.255.0

R2(config-if)# no shutdown

R3# conf t

R3(config)# interface loopback 3

R3(config-if)# ip address 172.16.3.1 255.255.255.0

R3(config-if)# interface loopback 103

R3(config-if)# ip address 192.168.103.1 255.255.255.0

R3(config-if)# interface fastethernet 0/0

R3(config-if)# ip address 172.16.23.3 255.255.255.0

R3(config-if)# no shutdown

R3(config-if)# interface serial 0/0/0

R3(config-if)# bandwidth 64

R3(config-if)# ip address 172.16.13.3 255.255.255.0

R3(config-if)# clock rate 64000

R3(config-if)# no shutdown

Step 2: Configure switch VLANs.

Switch(config)# vlan 12

Switch(config-vlan)# name R1-R2

Switch(config-vlan)# vlan 23

Switch(config-vlan)# name R2-R3

Switch(config-vlan)# exit

Switch(config)# interface fastEthernet 0/1

Switch(config-if)# description To R1 Fa0/0

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 12

Switch(config-if)# interface fastEthernet 0/2

Switch(configif)# description To R2 Fa0/0

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 12

Switch(config-if)# interface fastEthernet 0/3

Switch(configif)# description To R3 Fa0/0

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 23

Switch(config-if)# interface fastEthernet 0/4

Switch(configif)# description To R2 Fa0/1

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 23

Step 3: Configure RIP.

R1(config)# router rip

R1(config-router)# version 2

R1(config-router)# no auto-summary

R1(config-router)# network 172.16.0.0

R1(config-router)# network 192.168.101.0

R2(config)# router rip

R2(config-router)# version 2

R2(config-router)# no auto-summary

R2(config-router)# network 172.16.0.0

R2(config-router)# network 192.168.102.0

R3(config)# router rip

R3(config-router)# version 2

R3(config-router)# no auto-summary

R3(config-router)# network 172.16.0.0

R3(config-router)# network 192.168.103.0

R1# show ip route rip

R2# show ip route rip

R3# show ip route rip

Step 4: Configure OSPF.

R1(config)# interface loopback 1

R1(config-if)# ip ospf network point-to-point

R1(config-if)# interface loopback 101

R1(config-if)# ip ospf network point-to-point

R1(config-if)# router ospf 1

R1(config-router)# network 172.16.0.0 0.0.255.255 area 0

R1(config-router)# network 192.168.101.0 0.0.0.255 area 0

R2(config)# interface loopback 2

R2(config-if)# ip ospf network point-to-point

R2(config-if)# interface loopback 102

R2(config-if)# ip ospf network point-to-point

R2(config-if)# router ospf 1

R2(config-router)# network 172.16.0.0 0.0.255.255 area 0

R2(config-router)# network 192.168.102.0 0.0.0.255 area 0

R3(config)# interface loopback 3

R3(config-if)# ip ospf network point-to-point

R3(config-if)# interface loopback 103

R3(config-if)# ip ospf network point-to-point

R3(config-if)# router ospf 1

R3(config-router)# network 172.16.0.0 0.0.255.255 area 0

R3(config-router)# network 192.168.103.0 0.0.0.255 area 0

R1# show ip ospf neighbor

R1# show ip route

R2# show ip ospf neighbor

R2# show ip route

R3# show ip ospf neighbor

R3# show ip route

Step 5: Modify the routing protocol distance.

R1(config)# router rip

R1(config-router)# distance 100

R2(config)# router rip

R2(config-router)# distance 100

R3(config)# router rip

R3(config-router)# distance 100

R1# show ip route

R2# show ip route

R3# show ip route

R1# show ip protocols

Step 6: Modify distance based on route source.

R1(config)# router ospf 1

R1(config-router)# distance 85 192.168.100.0 0.0.3.255

R2(config)# router ospf 1

R2(config-router)# distance 85 192.168.100.0 0.0.3.255

R3(config)# router ospf 1

R3(config-router)# distance 85 192.168.100.0 0.0.3.255

R1# show ip route

R2# show ip route

R3# show ip route

R1# show ip protocols

Step 7: Modify distance based on an access list.

R1(config)# access-list 1 permit 172.16.0.0 0.0.255.255

R1(config)# router rip

R1(config-router)# distance 65 0.0.0.0 255.255.255.255 1

R2(config)# access-list 1 permit 172.16.0.0 0.0.255.255

R2(config)# router rip

R2(config-router)# distance 65 0.0.0.0 255.255.255.255 1

R3(config)# access-list 1 permit 172.16.0.0 0.0.255.255

R3(config)# router rip

R3(config-router)# distance 65 0.0.0.0 255.255.255.255 1

R1# show ip protocols

R1# show ip route

R2# show ip protocols

R2# show ip route

R3# show ip protocols

R3# show ip route

P 5 Configuring BGP with Default Routing

Step 1: Configure the hostname and interface addresses.

Router R1 (hostname ISP1) hostname ISP1

interface Lo0

description ISP1 Internet Network

ip address 10.1.1.1 255.255.255.0

interface Serial0/0/0

description ISP1 -> ITA

ip address 10.0.0.1 255.255.255.252

clock rate 128000

no shutdown

end

Router R2 (hostname ITA)

hostname ITA

interface Lo0

description Core router network link 1

ip address 192.168.0.1 255.255.255.0 !

interface Lo1

description Core router network link 2

ip address 192.168.1.1 255.255.255.0

interface Serial0/0/0

description ITA -> ISP1

ip address 10.0.0.2 255.255.255.252

no shutdown

interface Serial0/0/1

description ITA -> ISP2

ip address 172.16.0.2 255.255.255.252

clock rate 128000

no shutdown

end

Router R3 (hostname ISP2)

hostname ISP2

interface Lo0

description ISP2 Internet Network

ip address 172.16.1.1 255.255.255.0

interface Serial0/0/1

description ISP2 -> ITA

ip address 172.16.0.1 255.255.255.252

no shutdown

end

Step 2: Configure BGP on the ISP routers.

ISP1(config)# router bgp 200

ISP1(config-router)# neighbor 10.0.0.2 remote-as 100

ISP1(config-router)# network 10.1.1.0 mask 255.255.255.0

ISP2(config)# router bgp 300

ISP2(config-router)# neighbor 172.16.0.2 remote-as 100

ISP2(config-router)# network 172.16.1.0 mask 255.255.255.0

Step 3: Configure BGP on the ITA boundary router.

ITA(config)# router bgp 100

ITA(config-router)# neighbor 10.0.0.1 remote-as 200

ITA(config-router)# neighbor 172.16.0.1 remote-as 300

ITA(config-router)# network 192.168.0.0

ITA(config-router)# network 192.168.1.0

ITA# show ip route

Step 4: Verify BGP on the routers.

ITA# show ip bgp

ISP1# show ip bgp

ITA# show ip bgp neighbors

Step 5: Configure route filters.

ISP2# show ip rout

ITA(config)# access-list 1 permit 192.168.0.0 0.0.1.255

ITA# clear ip bgp *

ISP2# show ip route

ISP1# show ip route

Step 6: Configure primary and backup routes using floating static routes.

ITA# show ip route

ITA(config)# ip route 0.0.0.0 0.0.0.0 10.0.0.1 210

ITA(config)# ip route 0.0.0.0 0.0.0.0 172.16.0.1 220

ITA# show ip route

ISP1# config t

ISP1(config)# interface loopback 100

ISP1(config-if)# ip address 192.168.100.1 255.255.255.0

ITA# show ip route

ITA# ping

ITA# ping 192.168.100.1 source 192.168.1.1

Step 7: Configure primary and backup routes using a default network and a static route.

ITA(config)# no ip route 0.0.0.0 0.0.0.0 10.0.0.1 210

ITA(config)# no ip route 0.0.0.0 0.0.0.0 172.16.0.1 220

ISP1(config)# router bgp 200

ISP1(config-router)# network 192.168.100.0

ISP1(config-router)# end

ITA# show ip route

ITA(config)# ip default-network 192.168.100.0

ITA# show ip route

ITA(config)# ip route 0.0.0.0 0.0.0.0 172.16.0.1 220

ITA# show ip route

ITA# clear ip bgp 10.0.0.1

ITA# show ip route

P6 - Configuring OSPF for IPv6

P6 - Configuring OSPF for IPv6

Step 2: Configuring the hostname and loopback interfaces.

Router(config)# hostname R1

R1(config)# interface loopback0

R1(config-if)# ip address 10.1.1.1 255.255.255.0 \

R1(config-if)# ipv6 address FEC0::1:1/112

Router(config)# hostname R2

R2(config)# interface loopback0

R2(config-if)# ip address 10.1.2.1 255.255.255.0

R2(config-if)# ipv6 address FEC0::2:1/112

Router(config)# hostname R3

R3(config)# interface loopback0

R3(config-if)# ip address 10.1.3.1 255.255.255.0

R3(config-if)# ipv6 address FEC0::3:1/112

Step 3: Configure static IPv6 addresses.

R1(config)# interface serial0/0/0

R1(config-if)# ipv6 address FEC0::12:1/112

R1(config-if)# clockrate 64000

R1(config-if)# bandwidth 64

R1(config-if)# no shutdown

R1(config-if)# interface serial0/0/1

R1(config-if)# ipv6 address FEC0::13:1/112

R1(config-if)# bandwidth 64

R1(config-if)# no shutdown

R2(config)# interface serial0/0/0

R2(config-if)# ipv6 address FEC0::12:2/112

R2(config-if)# bandwidth 64

R2(config-if)# no shutdown

R3(config)# interface serial0/0/0

R3(config-if)# ipv6 address FEC0::13:3/112

R3(config-if)# clockrate 64000

R3(config-if)# bandwidth 64

R3(config-if)# no shutdown

R1# ping FEC0::12:2

R1# ping FEC0::13:3

R2# ping FEC0::12:1

R3# ping FEC0::13:1

Step 4: Change the link-local address on an interface.

R1# show ipv6 interface serial 0/0/0

R2# show ipv6 interface serial 0/0/0

R1(config)# interface serial0/0/0

R1(config-if)# ipv6 address FE80::1 link-local

R2(config)# interface serial0/0/0

R2(config-if)# ipv6 address FE80::2 link-local

R1# ping FE80::2

R2# ping FE80::1

R1# show ipv6 interface serial 0/0/0

R2# show ipv6 interface serial 0/0/0

Step 5: Configure EUI-64 addresses.

R2(config)# interface fastEthernet 0/0

R2(config-if)# ipv6 address FEC0:23::/64 eui-64

R2(config-if)# no shutdown

R3(config)# interface fastEthernet 0/0

R3(config-if)# ipv6 address FEC0:23::/64 eui-64

R3(config-if)# no shutdown

R2# show ipv6 interface fastEthernet 0/0

R2# show ipv6 interface brief

R2# ping FEC0:23::218:B9FF:FECD:BEF0

R3# ping FEC0:23::218:B9FF:FE92:28D8

Step 6: Enable IPv6 routing and CEF.

R1(config)# ipv6 unicast-routing

R1(config)# ipv6 cef

R2(config)# ipv6 unicast-routing

R2(config)# ipv6 cef

R3(config)# ipv6 unicast-routing

R3(config)# ipv6 cef

7: Configure OSPFv3.

R1(config)# interface loopback0

R1(config-if)# ipv6 ospf 1 area 0

R1(config-if)# interface serial0/0/0

R1(config-if)# ipv6 ospf 1 area 0

R1(config-if)# interface serial0/0/1

R1(config-if)# ipv6 ospf 1 area 0

R2(config)# interface loopback0

R2(config-if)# ipv6 ospf 1 area 0

R2(config-if)# interface serial0/0/0

R2(config-if)# ipv6 ospf 1 area 0

R2(config-if)# interface fastEthernet 0/0

R2(config-if)# ipv6 ospf 1 area 0

R3(config)# interface loopback0

R3(config-if)# ipv6 ospf 1 area 0

R3(config-if)# interface serial0/0/0

R3(config-if)# ipv6 ospf 1 area 0

R3(config-if)# interface fastEthernet 0/0

R3(config-if)# ipv6 ospf 1 area 0

R1# show ipv6 ospf neighbor

R2# show ipv6 ospf neighbor

R3# show ipv6 ospf neighbor

R1# show ipv6 route

R2# show ipv6 route

R3# show ipv6 route

P7 – Configuring Ether Channel

Step 1: Prepare the switches for the lab and Configure basic switch parameters.

DLS1# configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

DLS1(config)# interface range fastEthernet 0/7 - 12

DLS1(config-if-range)# switchport trunk encapsulation dot1q

DLS1(config-if-range)# switchport mode trunk

Step 3: Configure an EtherChannel with Cisco PAgP.

ALS1# show interfaces trunk

ALS1(config)# interface range fastEthernet 0/11 - 12

ALS1(config-if-range)# channel-group 1 mode desirable

Creating a port-channel interface Port-channel 1

ALS1(config)# interface port-channel 1

ALS1(config-if)# switchport mode trunk

ALS1# show etherchannel summary

ALS2(config)# interface range fastEthernet 0/11 - 12

ALS2(config-if-range)# channel-group 1 mode desirable

Creating a port-channel interface Port-channel 1

ALS2(config)# interface port-channel 1

ALS2(config-if)# switchport mode trunk

ALS2# show etherchannel summary

ALS1# show interfaces trunk

ALS1# show spanning-tree

Step 4: Configure an 802.3ad LACP EtherChannel.

ALS1(config)# interface range fastEthernet 0/7 - 8

ALS1(config-if-range)# channel-group 2 mode active

Creating a port-channel interface Port-channel 2

ALS1(config-if-range)# interface port-channel 2

ALS1(config-if)# switchport mode trunk

ALS1# show etherchannel summary

Step 5: Configure a Layer 3 EtherChannel.

DLS1(config)# interface range fastEthernet 0/11 - 12

DLS1(config-if-range)# no switchport

DLS1(config-if-range)# channel-group 3 mode desirable

Creating a port-channel interface Port-channel 3

DLS1(config-if-range)# interface port-channel 3

DLS1(config-if)# no switchport

DLS1(config-if)# ip address 10.0.0.1 255.255.255.0

DLS2(config)# interface range fastEthernet 0/11 - 12

DLS2(config-if-range)# no switchport

DLS2(config-if-range)# channel-group 3 mode desirable

Creating a port-channel interface Port-channel 3

DLS2(config-if-range)# interface port-channel 3

DLS2(config-if)# no switchport

DLS2(config-if)# ip address 10.0.0.2 255.255.255.0

DLS1# ping 10.0.0.2

DLS1# show etherchannel summary

Step 6: Configure load balancing.

DLS1# show etherchannel load-balance

EtherChannel Load-Balancing Operational State (src-mac):

Non-IP: Source MAC address

IPv4: Source MAC address

IPv6: Source IP address

ALS1(config)# port-channel load-balance src-dst-mac

ALS1# show etherchannel load-balance

EtherChannel Load-Balancing Operational State (src-dst-mac):

Non-IP: Source XOR Destination MAC address

IPv4: Source XOR Destination MAC address

IPv6: Source XOR Destination IP address

P8 - Inter-VLAN Routing with an External Router

Required Resources

• 2 routers (Cisco 1841 with Cisco IOS Release 12.4(24)T1 Advanced IP Services or comparable)

• 2 switches (Cisco 2960 with the Cisco IOS Release 12.2(46)SE C2960-LANBASEK9-M image or comparable)

Step 3: Configure the routers.

Router(config)# hostname ISP

ISP(config)# interface Loopback0

ISP(config-if)# ip address 200.200.200.1 255.255.255.0

ISP(config-if)# interface Serial0/0/0

ISP(config-if)# ip address 192.168.1.2 255.255.255.0

ISP(configif)# no shutdown

ISP(config-if)# exit

ISP(config)# ip route 172.16.0.0 255.255.0.0 192.168.1.1

Router(config)# hostname Gateway

Gateway(config)# interface Serial0/0/0

Gateway(config-if)# ip address 192.168.1.1 255.255.255.0

Gateway(config-if)# clockrate 64000

Gateway(config-if)# no shutdown

Gateway(config-if)# exit

Gateway(config)# ip route 0.0.0.0 0.0.0.0 192.168.1.2

Step 4: Configure the switches.

Switch(config)# hostname ALS1

ALS1(config)# interface vlan 1

ALS1(config-if)# ip address 172.16.1.101 255.255.255.0

ALS1(config-if)# no shutdown

ALS1(config-if)# exit

ALS1(config)# ip default-gateway 172.16.1.1

Switch(config)# hostname ALS2

ALS2(config)# interface vlan 1

ALS2(config-if)# ip address 172.16.1.102 255.255.255.0

ALS2(config-if)# no shutdown

ALS2(config-if)# exit

ALS2(config)# ip default-gateway 172.16.1.1

ALS1(config)# enable secret cisco

ALS1(config)# line vty 0 15

ALS1(config-line)# password cisco

ALS1(config-line)# login

ALS1(config-line)# end

ALS2(config)# enable secret cisco

ALS2(config)# line vty 0 15

ALS2(config-line)# password cisco

ALS2(config-line)# login

ALS2(config-line)# end

Step 5: Confirm the VLANs.

ALS1# show vlan

Step 6: Configure trunk links and EtherChannel on switches.

ALS1# configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

ALS1(config)# interface range fastEthernet 0/11 - 12

ALS1(config-if-range)# switchport mode trunk

ALS1(config-if-range)# channel-group 1 mode desirable

ALS1(config-if-range)# end

ALS2# configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

ALS2(config)# interface range fastEthernet 0/11 - 12

ALS2(config-if-range)# switchport mode trunk

ALS2(config-if-range)# channel-group 1 mode desirable

ALS2(config-if-range)# end

ALS1# show etherchannel 1 summary

Step 7: Configure VTP.

ALS2(config)# vtp mode client

Setting device to VTP CLIENT mode.

ALS1(config)# vtp domain SWLAB

Changing VTP domain name from NULL to SWLAB

%SW_VLAN-6-VTP_DOMAIN_NAME_CHG: VTP domain name changed to SWLAB.

ALS1(config)# vtp version 2

ALS1# show vtp status

ALS2# show vtp status

ALS1(config)# vlan 100

ALS1(config-vlan)# name Payroll

ALS1(config-vlan)# vlan 200

ALS1(config-vlan)# name Engineering

ALS2# show vlan brief

ALS1(config)# interface fastEthernet 0/6

ALS1(config-if)# switchport mode access

ALS1(config-if)# switchport access vlan 100

ALS1(config-if)# spanning-tree portfast

ALS2(config)# interface fastEthernet 0/6

ALS2(config-if)# switchport mode access

ALS2(config-if)# switchport access vlan 200

ALS2(config-if)# spanning-tree portfast

ALS1# show vlan brief

ALS2# show vlan brief

ALS1(config)# interface fastEthernet 0/1

ALS1(config-if)# switchport mode trunk

ALS1(config-if)# end

Step 10: Configure the Gateway router Fast Ethernet interface for VLAN trunking.

Gateway(config)# interface fastEthernet 0/0

Gateway(config-if)# no shut

The following is a sample configuration for the VLAN 1 subinterface.

Gateway(config)# interface fastEthernet 0/0.1

Gateway(config-subif)# description Management VLAN 1

Gateway(config-subif)# encapsulation dot1q 1 native

Gateway(config-subif)# ip address 172.16.1.1 255.255.255.0

Gateway(config-subif)# interface fastEthernet 0/0.100

Gateway(config-subif)# description Payroll VLAN 100

Gateway(config-subif)# encapsulation dot1q 100

Gateway(config-subif)# ip address 172.16.100.1 255.255.255.0

Gateway(config-subif)# interface fastEthernet 0/0.200

Gateway(config-subif)# description Engineering VLAN 200

Gateway(config-subif)# encapsulation dot1q 200

Gateway(config-subif)# ip address 172.16.200.1 255.255.255.0

Gateway(configsubif)# end

Gateway# show ip interface brief

Gateway# show interfaces description

Gateway# show vlans

P9 – Modifying Default Spanning Tree

Required Resources

• 2 switches (Cisco 2960 with the Cisco IOS Release 12.2(46)SE C2960-LANBASEK9-M image or

comparable)

• 2 switches (Cisco 3560 with the Cisco IOS Release 12.2(46)SE C3560-ADVIPSERVICESK9-M

image or comparable)

• 1 PC (optional) attached to switch ALS1.

Step 1: Prepare the switches for the lab.

DLS1(config)# interface range fastEthernet 0/7 - 12

DLS1(config-if-range)# switchport trunk encapsulation dot1q

DLS1(config-if-range)# switchport mode trunk

Step 2: Display default spanning tree information for all switches.

DLS1# show spanning-tree

DLS2# show spanning-tree

ALS1# show spanning-tree

ALS2# show spanning-tree

Switch# show spanning-tree

Switch# conf t

Switch(config)# interface range fastEthernet 0/1-24

Switch(config-if-range)# no shutdown

Switch(config-if-range)# end

Switch# show spanning-tree

DSL1# show interfaces trunk

Step 3: Configure specific switches to be primary and secondary root.

DLS2# debug spanning-tree events

Spanning Tree event debugging is on

DLS1(config)# spanning-tree vlan 1 root primary

ALS1(config)# spanning-tree vlan 1 root secondary

DLS1# show run | include span spanning-tree mode pvst

ALS1# show run | include span

DLS1# show spanning-tree

Step 4: Change the root port using the spanning-tree port-priority command.

DLS2# show spanning-tree

DLS1# show spanning-tree

DLS1(config)# int fastEthernet 0/12

DLS1(config-if)# spanning-tree port-priority 112

DLS2# show spanning-tree

DLS1# show spanning-tree

Step 5: Configure PortFast on an access port.

ALS1(config)# interface fastEthernet 0/6

ALS1(config-if)# shutdown

ALS1# debug spanning-tree events

ALS1(config)# interface fastEthernet 0/6

ALS1(config-if)# switchport mode access

ALS1(config-if)# no shut

ALS1(config-if)# end

ALS1(config)# interface fastEthernet 0/6

ALS1(config-if)# shutdown

ALS1(config)# interface fastEthernet 0/6

ALS1(config-if)# spanning-tree portfast

ALS1(config-if)# no shutdown

ALS1(config-if)# end

ALS1# undebug all

Step 6: Change root port using the spanning-tree cost command.

ALS2# show spanning-tree

ALS2(config)# interface fastEthernet 0/10

ALS2(config-if-range)# spanning-tree cost 10

ALS2# show spanning-tree

P 9 Modifying Default Spanning Tree.txt

Displaying P 9 Modifying Default Spanning Tree.txt.